|  | HAVERING MINIATURE RAILWAY CLUB |  |
| --- | --- | --- |
| Rolling Stock Checklist – Must be carried out a minimum of every 12 months |  |  |  |

| COACH DESCRIPTION/NAME |  |
| --- | --- |

## TWO COMPETENT PERSONS  TO CARRY OUT INSPECTION

| Name: |  |  | Date: |  |
| --- | --- | --- | --- | --- |
| Name: |  |  |  |  |

## BOGIES

No damage to wheel treads. Assessment of wheel wear - Coning, flange depth and radius Wheels are secure on axles – gaps, lateral movement etc. Back to back should be around 172mm. Check bearings are tight on axles – no lateral movement. Wheels turn freely. All springs are present and correctly located. No excessive movement between axle boxes and keeps (independent sprung axles). Full travel of axle boxes in keep or suspension – one person watches while other person brings weight to bear on the coach to check the travel. All bolts in place and not loose (spanner only required to check if any bolts are found to be loose). Bogie swivel – check bogies can rotate freely. Bogie Pivot – Tight enough to restrict tilting but not too tight to restrict rotation. Oil/grease to axle boxes and keeps. ## BODY WORK Check no snags, flaking paint or sharp points that can cause injury. Structurally sound. Seats are secure and do not move (Mardyke Coaches). Bogie cross plates are tight Couplings are secure and in good condition. ## BRAKES Vacuum system will pressurise and hold at 16” hg. Brakes work sufficiently to lock or nearly lock wheels when coach unloaded state. Visually inspect hoses for abrasions and wear. ## NOTES COACH IS FIT/NOT FIT (delete as appropriate) FOR PUBLIC RUNNING..

Signatures